NATIONAL UNIVERSITY OF TECHNOLOGY ISLAMABAD

DEPARTMENT OF COMPUTER SCIENCE

DIGITAL LOGIC DESIGN LAB

| Name | Bisma Fatima |
| --- | --- |
| Roll No | 36 |
| Batch | 24 |
| Section | A |
| Lab Number | 06 |
| Instructor Name | Sir Haseeb Miss Tehreem khalil |
| Date of Submission | April 07TH, 2025 |
| Topic Covered | Implementing Adders and Subtractors |

Experiment 6

Implementing Adders and Subtractors

Objective

- To experimentally verify the operation of basic adders and subtractors

- To design various configurations of adders and subtractors

Tools/Equipment Requirement

- ICs as required

- 7486 (XOR gate)

- 7408 (AND gate)

- 7404 (NOT gate)

- 7432 (OR gate)

- Digital Electronics trainer.

Theoretical Explanation

An adder is a digital circuit that performs addition of numbers. The half adder adds two binary digits called as augend and addend and produces two outputs as sum and carry; XOR is applied to both inputs to produce sum and AND gate is applied to both inputs to produce carry.

The half subtractor is a combinational circuit which is used to perform subtraction of two bits. It has two inputs, the minuend  and subtrahend  and two outputs the difference  and borrow out.

A full adder is a logical circuit that performs an addition operation on three one-bit binary numbers. The full adder produces a sum of the three inputs and carry value.

The following experiment involves the design of adder/subtractor circuits using the formation of Boolean expression using truth tables.

Half Adder

- Complete the truth table.

- Write the logic equations for Sum and Carry.

- Draw the complete logic diagram for the Half Adder.

- Verify your results practically.

Observations/Truth Table

|  | Truth Table |  |  | Practical Values |  |
| --- | --- | --- | --- | --- | --- |
| A | B | SUM | CARRY | SUM | CARRY |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 0 | 1 |

Equations

Sum    = A ⊕ B                          Carry = A * B

Procedure

- Take two binary inputs, A and B.

- Calculate the sum (S) using XOR operation: S = A ⊕ B.

- Calculate the carry (C) using AND operation: C = A * B.

Circuit/Logic Diagram

Explanation of circuit

In above figures we have used 2 IC chips [ XOR ,AND]. XOR chip is use to make the logical circuit of SUM And AND chip is use to make the logical circuit of CARRY. We have taken 2 inputs [A,B]. Also we shows two output wires , 1 show output for SUM and other shows output for CARRY.

Half Subtractor

- Complete the truth table.

- Write the logic equations for Difference and Borrow.

- Draw the complete logic diagram for the Half Subtractor.

- Verify your results practically.

Observations/Truth Table

|  | Truth Table |  |  | Practical Values |  |
| --- | --- | --- | --- | --- | --- |
| A | B | Borrow | Difference | Borrow | Difference |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 0 | 0 |

D    = A ⊕ B                                B    =      A’*B

Procedure

- Take two binary inputs, A and B.

- Calculate the difference using XOR operation: S = A ⊕ B.

- Calculate the borrow using AND,NOT operations: C = A’* B.

Circuit/Logic Diagram

Explanation of circuits

In above figures we have used 3 IC chips [ XOR ,AND,NOT]. XOR chip is use to make the logical circuit of DIFFERRNCE And AND,NOT chips are use to make the logical circuit of BORROW. We have taken 2 inputs [A,B]. Also we shows two output wires , 1 show output for DIFFERENCE and other shows output for BORROW.

Full Adder

- Draw the complete logic diagram.

- Write the logic equations for Sum and Carry out.

- Complete the Truth Table.

- Implement the circuit on trainer.

- Verify your results practically.

Observations/Truth Table

|  |  | Truth Table |  |  | Practical Values |  |
| --- | --- | --- | --- | --- | --- | --- |
| A | B | Ci | SUM | Co | SUM | Co |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Co   = (A·B) + (B·Ci) + (A·Ci)

Full adder sum equation:

S = A’B’Ci + A’BCi’+AB’Ci’+ABCi

S=A’(B’Ci+BCi’)+A(B’Ci’+BCi)

S=A’(B ⊕ Ci )+A((B  ⊕ Ci)’)

S=(A ⊕ B )  ⊕ C

Full adder carry equation:

Co=A’.B.Ci+A.B’.Ci+A.B.Ci’+A.B.Ci                                                B.Ci

by using k-map we simplify it;

| A\BCi | B’Ci’ | B’Ci | BCi | BCi’ |
| --- | --- | --- | --- | --- |
| A’ |  |  | 1 |  |
| A |  | 1 | 1 | 1 |

A.Ci                                                                           A.B

Co=A.B+A.Ci+B.Ci

Procedure

- Take three binary inputs: A, B, and Cin (carry-in).

- Calculate the sum (S) using XOR operation: S = A ⊕ B ⊕ Cin.

- Calculate the carry (Cout) using the following formula: Cout = (A * B) + (A * Cin) + (B * Cin).

Circuit/Logic Diagram

- Logical diagram for SUM

- Logical diagram for CARRY

Explanation of circuit

In above figures we have used 3 IC chips [ XOR ,AND,OR]. XOR chip is use to make the logical circuit of SUM And AND,OR chips are use to make the logical circuit of CARRY. We have taken 3 inputs [A,B,Ci]. Also we shows two output wires , 1 show output for SUM and other shows output for CARRY.

Post Lab Task

- Design a 4-bit Ripple Carry Adder using Full Adders.

- Draw its Truth table and logic Diagram using CircuitVerse

Concept

A Ripple Carry Adder (RCA) is essentially a series of full adders connected in a sequence. In this setup, the carry-out from one bit is passed on as the carry-in to the next bit.

Structure

- It consists of four full adders.

- Input: Two 4-bit numbers, A[3:0] and B[3:0], along with an initial carry-in (C0).

- Output: A 4-bit Sum and the final carry-out (C4).

Observations/Truth Table

| A3 | A2 | A1 | A0 | B3 | B2 | B1 | B0 | Cin | Sum3 | Sum2 | Sum1 | Sum0 | Cout |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Equations:

Sum Bits

Sum3

- Sum3 = (A3 XOR B3) XOR Cin

Sum2

- Sum2 = (A2 XOR B2) XOR (Carry from Sum3)

Sum1

- Sum1 = (A1 XOR B1) XOR (Carry from Sum2)

Sum0

- Sum0 = (A0 XOR B0) XOR (Carry from Sum1)

Carry Out

Cout = (A3 AND B3) OR (Cin AND (A3 XOR B3)) OR (Carry from Sum3)

I've used the following notation:

XOR: ⊕

AND: ⋅ or AND

OR: + or OR

Explanation

4-Bit Binary Adder Explanation

- Inputs

  - A: 4-bit binary number (A3, A2, A1, A0)

  - B: 4-bit binary number (B3, B2, B1, B0)

  - Cin: Input carry

- Outputs

  - Sum: 4-bit sum (Sum3, Sum2, Sum1, Sum0)

  - Cout: Carry-out

- Operation

- The adder calculates the sum bit-by-bit, considering the carry from the previous bit. The carry-out indicates whether there's a carry-over from the most significant bit.

- Truth Table

- The table shows all possible input combinations and their corresponding sum and carry-out values.

Circuit/Logic Diagram

First Image – Full Adder Circuit

This full adder circuit diagram shows its internal operation:

- Inputs

- A

- B

- Cin (carry-in)

- Outputs

- Sum (result of A, B, and Cin addition)

- Carry (output carry)

- Logic Operations

- XOR gates calculate the sum (A ⊕ B ⊕ Cin)

- AND and OR gates compute the carry ((A AND B) OR (B AND Cin) OR (A AND Cin))

This diagram provides a detailed, logic-level view of a single-bit full adder's operation.

Second Image – 4-Bit Ripple Carry Adder

This circuit connects four full adders in a series:

How it works

- Each full adder processes one bit, starting from the least significant bit (LSB) and moving to the most significant bit (MSB).

- The carry output from each stage is passed on to the next stage.

- The final result includes a 4-bit sum (S0-S3) and an overflow indicator (shown by a red LED).

This setup is called a ripple-carry adder.

Learning Outcomes

- Experience with arithmetic logic circuits.

- Learned to apply truth tables to Boolean expressions.

- Created logic diagrams from Boolean expressions.

- Understood full adder structure in ripple-carry systems.

- Developed practical skills with digital trainer boards and integrated circuits.