NATIONAL UNIVERSITY OF TECHNOLOGY ISLAMABAD

DEPARTMENT OF COMPUTER SCIENCE

DIGITAL LOGIC DESIGN LAB

| Name | Bisma Fatima |
| --- | --- |
| Roll No | 36 |
| Batch | 24 |
| Section | A |
| Lab Number | 04 |
| Instructor Name | Sir Haseeb Miss Tehreem khalil |
| Date of Submission | March 24TH, 2025 |
| Topic Covered | OEL 01 |

Experiment 5

Designing a Combinational Circuit using Logic Gates

(Open Ended Lab)

Objective

- Design, construct and test combinational circuits using logic gates.

Design Problem

Design a combinational circuit with four inputs A, B, C and D and one output F. F is to be equal to 1 when A = 1, provided that B = 0, or when B = 1, provided that either C or D is equal to 1. Otherwise, the output is equal to 0.

Design, draw and implement the circuit using any type of logic gates with minimum number of ICs.

- Obtain the truth table of the circuit and simplify the output function.

- List out the ICs required.

- Write down the steps of the procedure.

Tools/Equipment Requirement

Identify and list down

The circuit requires basic logic gates. The standard ICs needed are:

● 7404 (NOT gate) – to invert B.

● 7408 (AND gate) – for AND operations.

● 7432 (OR gate) – for OR operations.

Activity Time Boxing

| Task No. | Activity Name | Activity Time |
| --- | --- | --- |
| 1 | Explaining Problem Statement | 10 ~ 15 mins |
| 2 | Building up logic | 15 ~ 20 mins |
| 3 | Draw a logic diagram | 10 ~ 15mins |
| 4 | Identify the equipment required | 10 ~ 15 mins |
| 5 | Devising a procedure to get the required output | 90~95min |
| 6 | Output Verification and Viva | 15 ~ 20 mins |
| Total Time: 180 |  |  |

Understanding the Problem Statement:

We need to design a combinational circuit with four inputs (A, B, C, D) and one output (F). The output F = 1 under the following conditions:

1. When A = 1, provided that B = 0.

2. When B = 1, provided that either C or D is 1.

3. Otherwise, F = 0.

This means the Boolean function should be designed based on these conditions.

Constructing the Boolean Expression:

1. The first condition (A = 1, B = 0) can be written as:

A⋅B`

2. The second condition (B = 1 and (C = 1 or D = 1)) can be written as:

B`⋅(C + D)

3. Combining both expressions using OR (since either condition can make F = 1):

F=(A⋅B`)+A(B⋅(C+D))

Simplified:

1. The given Boolean expression is:

- F=(A⋅ B`)+A(B⋅ (C+D))

- F=(A⋅ B`)+(A⋅ B⋅ C)+(A⋅ B⋅ D)

- F=A(B`+B⋅ C+B⋅ D)          //simplified form

Procedure:

Step 1: Truth Table Generation

- Generate all possible input combinations (16 cases) for inputs A, B, C, and D.

- Calculate the output F for each case using the given Boolean equation.

Step 2: Boolean Expression Simplification

- Minimize the Boolean equation, if possible, to reduce complexity.

Step 3: Circuit Design

- Design a digital circuit diagram based on the simplified Boolean equation.

- Utilize logic gates (AND, OR, NOT, etc.) to implement the Boolean function.

Step 4: Circuit Assembly and Simulation

- Assemble the designed circuit on a breadboard or simulate it using software.

- Place integrated circuits (ICs) and connect them according to the circuit diagram.

Step 5: Circuit Verification

- Apply all 16 test cases from the truth table to the assembled or simulated circuit.

- Compare the actual output with the expected output from the truth table to verify the circuit's correctness.

Logic/Circuit Diagram

Circuit Verse Logic Diagram:

Circuit:

Observations/Truth Table

| INPUTS |  |  |  | Expression (Solve theoretically) |
| --- | --- | --- | --- | --- |
| A | B | C | D | F |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Learning Outcomes

- Learn Boolean algebra

- Design digital circuits

- Simplify circuits

- Build and test circuits